MODULE REPORT

PROCESSING ELEMENT
FOR A WEIGHT-STATIONARY
SYSTOLIC ARRAY

Design and Operation of a Single 8-bit
Multiply–Accumulate Processing Element

──────────────────────────────────

Submitted by

[ Name 1 ]

[ Name 2 ]

[ Name 3 ]

Guide

[ Guide Name ]

[ Department ]

[ Institution ]

[ Month, Year ]

## 1.  Overview

A Processing Element (PE) is the basic computing cell of a systolic array. It is a small synchronous circuit that performs one multiply–accumulate (MAC) operation per clock cycle and then passes its data on to its neighbouring cells. A complete systolic array contains nothing but copies of this one cell wired together, so the behaviour of the whole machine follows entirely from the behaviour of a single PE.

The PE described here is designed for a weight-stationary array. It holds one 8-bit signed weight in an internal register for the whole of a computation, multiplies that weight by an activation arriving from its left neighbour, adds the product to a partial sum arriving from the cell above, and passes the result to the cell below. It contains no instruction memory, no decoder and no addressable storage — its behaviour is fixed by its wiring and modulated only by two control signals.

## 2.  Role in the Systolic Array

Matrix multiplication is defined element-wise as

Cᵢⱼ = Σ Aᵢₖ · Bₖⱼ

which is a sum of products — exactly one MAC per term. A systolic array maps this directly onto hardware: each PE evaluates one term of the sum, and the summation itself is performed by physically passing the partial sum from one PE down to the next. No accumulator is ever addressed and no intermediate result is written to memory.

Three streams therefore meet inside every PE. The weight is stationary and stays in the cell. The activation moves horizontally, entering from the left and leaving to the right. The partial sum moves vertically, entering from the top and leaving at the bottom. Because the streams travel in different directions, an operand meets a different partner in every cell it visits — which is how a single value fetched once from memory contributes to many different results.

## 3.  The Multiply–Accumulate Operation

The single arithmetic operation performed by the PE is

Psum = Psum + (A × B)

or, in terms of the actual signal names of this module,

psum_out = psum_in + (weight_q × a_in)

This is the complete functional specification of the cell. Everything else — the registers, the multiplexers, the control signals — exists only to deliver the right operands to this equation at the right time and to move the result to the right place. One PE performs one MAC per cycle; a 4×4 array of them performs sixteen.

## 4.  Interface

psum_in [31:0]
                    │
                    ▼
     ┌────────────────────────────┐
     │                            │
 clk─►                            │
rst_n─►     PROCESSING            ├─► valid_out
wload─►        ELEMENT            │
  v_in─►                          ├─► a_out [7:0]
  a_in─►    IN_W = 8              │
 [7:0] │    ACC_W = 32            │
     └─────────────┬──────────────┘
                   │
                   ▼
             psum_out [31:0]

Fig. 1  Pin diagram of the processing element.

Table 1  Port description of the processing element.

| Signal | Dir / Width | Function |
| --- | --- | --- |
| clk | in, 1 | Global clock; all registers update on the rising edge |
| rst_n | in, 1 | Active-low synchronous reset; clears all registers |
| wload | in, 1 | 1 = weight-load phase, 0 = compute phase |
| valid_in | in, 1 | 1 = a_in carries a valid activation this cycle |
| a_in | in, 8 | Signed activation from the left neighbour |
| a_out | out, 8 | Registered activation to the right neighbour |
| valid_out | out, 1 | Registered valid flag to the right neighbour |
| psum_in | in, 32 | Signed partial sum from above; weight during wload |
| psum_out | out, 32 | Signed partial sum to below; weight during wload |

## 5.  Internal Architecture

psum_in[31:0]            a_in[7:0]
        │                       │
   ┌────┴─────┬─────────┐       ├──────┐
   │ [7:0]    │         │       │      ▼
   ▼          │         │       │  ┌───────┐
┌────────┐    │         │       │  │  a_q  │
│weight_q│◄── wload     │       │  └───┬───┘
└───┬────┘              │       │      ▼
    ├──────────┐        │       │    a_out
    ▼          │        │       │
┌─────────────────┐     │       │
│  MUL  8×8 → 16  │◄────┼───────┘
└────────┬────────┘     │
         │ sign-extend  │
         ▼              │
┌─────────────────┐     │
│   ADD  32-bit   │◄────┤
└────────┬────────┘     │
         ▼              │
┌─────────────────┐     │
│ MUX ◄ valid_in  │◄────┘
└────────┬────────┘
         ▼
┌─────────────────┐
│     psum_q      │
└────────┬────────┘
         ▼
┌─────────────────┐
│  MUX ◄ wload    │◄── weight_q (sign-extended)
└────────┬────────┘
         ▼
   psum_out[31:0]

  valid_in ──►[ valid_q ]──► valid_out

Fig. 2  Internal datapath of the processing element.

The cell contains four registers and three combinational blocks. There is exactly one register between any input and any output, so the PE adds exactly one cycle of latency to every path through it.

### weight_q — the stationary weight

An 8-bit register that loads from psum_in[7:0] on any rising clock edge on which wload is asserted, and holds its value otherwise. Because the register has an explicit enable, the compute traffic that later passes over the same psum wires cannot disturb it. This hold behaviour is what makes the architecture weight-stationary, and it is the single most important property of the cell. Only the low eight bits of the 32-bit bus are captured; the upper twenty-four are ignored.

### a_q and valid_q — the activation pipeline

a_q captures a_in on every clock edge and drives a_out; valid_q does the same for valid_in. Their purpose is to break the horizontal combinational path, so that an activation advances exactly one PE per cycle regardless of how wide the array is. valid_q travels alongside a_q so that the qualifier always arrives with the activation it qualifies.

Note that the multiplier is driven by a_in, the incoming value, not by a_q. The PE multiplies the activation present on its input during the current cycle and simultaneously registers that same value for its neighbour. Multiplying a_q instead would delay the arithmetic by one cycle relative to the data movement and destroy the alignment between activations and partial sums.

### psum_q — the accumulator

A 32-bit register that loads psum_in + weight_q × a_in when valid_in is high, and psum_in unchanged when it is low. It never holds its previous value: the accumulation is not local to the cell but distributed down the column, with each PE adding its contribution to a sum that is passing through. This is what allows a new independent dot product to enter the column on every single cycle.

## 6.  Weight Loading and the Shared psum Path

The psum wires carry two entirely different kinds of information at two different times. During the weight-load phase (wload = 1) they carry weights being shifted down the column; during the compute phase (wload = 0) they carry partial sums. The output multiplexer selects accordingly: it drives the sign-extended contents of weight_q when wload is high, and psum_q when wload is low.

The benefit is architectural rather than merely economical. Because weight_q drives psum_out during a load, each column of the array behaves as a vertical shift register, and all the weights of a column can be pushed in one per cycle through a single port at the top of the array. An n × n array therefore needs only n input ports to load n² weights. Without this time-sharing, weight distribution would dominate the wiring of the chip.

The sign extension in this path matters. A weight of −5, held as 8'hFB, must appear on the 32-bit bus as 32'hFFFF_FFFB and not as 32'h0000_00FB; otherwise the PE below would receive +251 instead of −5.

## 7.  Cycle-by-Cycle Operation

The behaviour of the cell is best seen through an example. The weight −3 is loaded, after which activations arrive with incoming partial sums.

Table 2  Operation of one PE with weight −3 loaded in cycle 0.

| Cyc | wl | v | a_in | psum_in | psum_out after edge |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | — | −3 | −3 (weight passes down) |
| 1 | 0 | 1 | 4 | 100 | 100 + (−3×4) = 88 |
| 2 | 0 | 1 | 5 | 200 | 200 + (−3×5) = 185 |
| 3 | 0 | 0 | 9 | 300 | 300 (unchanged) |
| 4 | 0 | 1 | 6 | 300 | 300 + (−3×6) = 282 |

Cycle 0 loads the weight, and psum_out carries the weight itself so that the PE below can capture it on the next cycle. Cycles 1 and 2 are normal MAC operations. Cycle 3 shows the importance of the valid gate: although a_in carries the value 9, valid_in is low, so the partial sum passes through untouched. Cycle 4 resumes normal operation. Throughout, weight_q never changes — the weight is stationary.

│ 0 │ 1 │ 2 │ 3 │ 4 │
  clk   ─┐ ┌─┐ ┌─┐ ┌─┐ ┌─┐ ┌─
         └─┘ └─┘ └─┘ └─┘ └─┘

  rst_n ──────────────────────

  wload ──┐
          └───────────────────

  v_in  ────┐       ┌─┐
            └───────┘ └───────

  a_in  ──╳ ─ ╳ 4 ╳ 5 ╳ 9 ╳ 6 ╳

  psum_in ╳ −3╳100╳200╳300╳300╳

  psum_out╳ −3╳ 88╳185╳300╳282╳
           ▲   ▲       ▲
        weight MAC   bubble
         out        (v_in=0)

Fig. 3  Waveform of the sequence in Table 2. Outputs change one cycle after the inputs that produce them.

## 8.  Connection to Neighbouring Cells

psum_in        psum_in
       │              │
       ▼              ▼
   ┌────────┐    ┌────────┐
──►│ PE i,j ├───►│PE i,j+1├──►
   └───┬────┘    └───┬────┘
       ▼             ▼
   ┌────────┐    ┌────────┐
──►│PE i+1,j├───►│PE i+1, ├──►
   └───┬────┘    └───┬────┘
       ▼             ▼
   psum_out       psum_out

Fig. 4  Nearest-neighbour interconnection between cells.

From the left — a_in and valid_in come from the a_out and valid_out of the PE to the left; cells in column 0 take them from the array boundary.

To the right — a_out and valid_out drive the next PE. In the last column they are unconnected — the activation has been fully used.

From above — psum_in comes from the psum_out of the PE above; the top row takes it from the array boundary (zero, or a weight during load).

To below — psum_out drives the PE below. In the bottom row it is the completed dot product leaving the array.

Every one of these connections is registered inside the cell, so the wire between two adjacent PEs is the only combinational path between them. This is what allows the array to be enlarged without any reduction in clock frequency.

## 9.  Design Considerations

### Signed arithmetic

Both operands are 8-bit two's-complement values in the range −128 to +127, so the multiplier must sign-extend them before forming partial products. Treating a negative weight as unsigned would interpret −1 as +255 and corrupt the result completely.

### Product and accumulator width

The product of two 8-bit signed values needs 16 bits: the extreme case (−128) × (−128) = +16 384 does not fit in 15. The product is sign-extended to 32 bits before the add. The 32-bit accumulator is deliberately generous — it leaves 16 guard bits, enough for 2¹⁶ worst-case accumulations, far beyond what a small array needs.

### Valid gating

The MAC is performed only when valid_in is asserted; otherwise psum_in passes through unchanged. This is essential during pipeline fill and drain, when some cells have no meaningful activation. Without the gate, the array would accumulate the products of whatever residual values happened to sit on the activation wires and silently corrupt every result.

### Synchronous reset

rst_n is active-low and synchronous: all four registers clear on a rising clock edge, not immediately. A synchronous reset avoids the recovery/removal timing problems of an asynchronous one and costs nothing here, since the array is never required to reset without a running clock.

### Critical path

The longest combinational path is one 8×8 multiplier followed by one 32-bit adder. It does not lengthen as the array grows, because every inter-cell connection is registered. If a higher clock frequency were needed, a pipeline register could be inserted between the multiplier and the adder at the cost of one extra cycle of latency.

## 10.  Functional Verification

Because the whole array is a replication of this one cell, a fault in the PE appears everywhere at once. The module is therefore verified on its own, against a golden reference model that recomputes psum + w × a independently, before being instantiated. Each test below is written to catch one specific class of design fault.

Table 3  Verification categories and the fault each is designed to catch.

| Test | Fault it catches |
| --- | --- |
| Reset | Asynchronous reset, registers that fail to clear, X values escaping reset |
| Weight load | Capturing more than 8 bits of psum_in; zero-extending instead of sign-extending the weight |
| Stationarity | A weight disturbed by compute traffic on the shared psum bus |
| Hop timing | A combinational (0-cycle) or over-registered (2-cycle) activation path; a_out and valid_out skewed apart |
| MAC | Wrong operand pairing, missing accumulate, unsigned multiply, product truncated before the add |
| Bubble | An accumulator that ignores valid_in and MACs on every cycle |
| Signed | Zero-extension of operands; mishandling of a negative incoming partial sum |
| Boundary | Truncation at 127×127, (−128)×(−128) and (−128)×127; a narrow accumulator dropping upper bits |
| Transitions | A weight that arrives one cycle late; residue left in psum_q by a load cycle |

Two conventions keep the testbench itself trustworthy. Stimulus is driven on the falling clock edge and outputs are sampled after the rising edge, so driving and checking can never race. And no expected value is ever typed by hand — every one is produced by the golden function, so a mistake in the testbench cannot accidentally agree with the same mistake in the design.

The bubble test deserves particular mention. It drives deliberately non-zero junk (8'h7F) on a_in while valid_in is low, with a non-zero weight loaded. Using a_in = 0 instead would pass even if the valid gate were missing entirely, because the product would be zero either way — a test that cannot fail is worse than no test at all.

## 11.  Summary

The processing element is a small synchronous cell containing one 8×8 signed multiplier, one 32-bit adder and four registers: weight_q for the stationary coefficient, a_q and valid_q for the horizontal activation pipeline, and psum_q for the vertical partial-sum pipeline. It computes psum_out = psum_in + weight_q × a_in whenever valid_in is asserted and passes the partial sum through unchanged otherwise. weight_q loads only when wload is asserted, which is what makes the design weight-stationary, and the psum wires are time-shared between weight distribution and partial-sum accumulation so that an n × n array can be loaded through only n ports. Because every interface is registered, the cell adds exactly one cycle of latency in each direction and can be replicated indefinitely without lengthening the critical path.